/********************************************************************************************

File: PWM.c,Author: www.spjsystems.com,Date: September 2014

Description   : This programm is related to DC on PWM.

**********************************************************************************************/

#include <p18f4520.h>

#include <delays.h>

#pragma config OSC = HS         // High-speed oscillator

#pragma config  WDT = OFF       // Watchdog Timer disabled

#pragma config LVP = OFF        // Low-voltage Programming disabled

#pragma config PBADEN = OFF

void main()

{

//RC1= interfacing with STK

//RC3= interfacing with Ultralite

unsigned char   dc ;

TRISC = 0 ;                     // set PORTC as output RC1,RC2,RC3 Configure as a PWM

PORTC = 0 ;                     // clear PORTC

/*

* configure CCP module as 10000 Hz PWM output

*/

PR2 = 0b01111100 ;

T2CON = 0b00000101 ;

CCP1CON = 0b00001100 ;

CCP2CON = 0b00111100 ;

for(;;)                         // forever

{

/*

* PWM resolution is 10 bits

* don't use last 2 less significant bits CCPxCON,

* so only CCPRxL have to be touched to change duty cycle

*/

for(dc = 0 ; dc < 128 ; dc++)

{

CCPR1L = dc ;

CCPR2L = 128 - dc ;

Delay10KTCYx(50);//200mS

}

for(dc = 127 ; dc > 0 ; dc--)

{

CCPR1L = dc ;

CCPR2L = 128 - dc ;

Delay10KTCYx(50);//200mS

}

}

}