/***************************************************************************

File: Combine.c,Author: www.spjsystems.com,Date: September 2014

Description   : This file contains Interface With Relay,Buzzer,Switch,Led's.

A. when button 1 is pressed relay and buzzer is turned ON and LED’s start chasing from left to right

B. when button 2 is pressed relay and buzzer is turned OFF and Led start chasing from right to left

***************************************************************************/

#include <p18f4520.h>

#include <delays.h>

#define BUZZER  PORTAbits.RA3 	//Buzzer connected to PORTA 3rd PIN

#define SWITCH0 PORTBbits.RB0	//Switch0 connected to PORTB 0th PIN

#define SWITCH1 PORTBbits.RB1	//Switch1 connected to PORTB 1st PIN

void main(void)

{

TRISA = 0x00;		// RA3,OutPut Direction

TRISB = 0xff;		// RB0,B1 Input Direction

TRISD = 0x00;		// [RD0-3=LED's][RD4,5=Relay1,2] OutPut Direction

PORTD = 0xff;	    // [RD0-3=LED's][RD4,5=Relay1,2] Initialise as 0xff

while (1)

{

if(!SWITCH1)      // Condition for 1st switch

{

while (1)

{

BUZZER =1;			// Buzzer On

PORTD = 0x37;		// (Relay1=1,Relay2=1) &  (LED's sequence Left to Right=0111=7)

Delay10KTCYx(100);	// 400mSDelay

PORTD = 0x3B;		// (LED's sequence Left to Right=1011=B)

Delay10KTCYx(100);

PORTD = 0x3D;

Delay10KTCYx(100);

PORTD = 0x3E;

Delay10KTCYx(100);

if(!SWITCH0)		// check if 2nd switch is pressed

break;

}

}

else if(!SWITCH0)	// Condition for 2nd switch

{

while (1)

{

BUZZER =0;			// Buzzer Off

PORTD = 0xcE;		// (Relay1=0,Relay2=0) &  (LED's sequence Right to Left=1110=E)

Delay10KTCYx(100);

PORTD = 0xcD;		// LED's sequence Right to Left=1101=D

Delay10KTCYx(100);

PORTD = 0xcB;

Delay10KTCYx(100);

PORTD = 0xc7;

Delay10KTCYx(100);

if(!SWITCH1)		// check if 1st switch is pressed

break;

}

}

}

}