/********************************************************************************************

File: TimerInterrupt.c,Author: www.spjsystems.com,Date: September 2014

Description   : This programm is related to led blinking on port D.

**********************************************************************************************/

/*

Cal = for 20MHz oscillator and 256 prescalar, the effective instruction execution speed is (4*256)/20×10^6 which will give you 51.2uS (micro second) per instruction.

So for 1 second  19625(it is 1/51.2uS).

Since we know interrupt happens only when the timer buffer overflows,

for a 16 bit timer the preset value to achieve 1 second for overflow will be 65535 – 19625(0x4c4b)  =  0xb3b4.

*/

#include <p18f4520.h>

#pragma config OSC = HS         // High-speed oscillator

#pragma config  WDT = OFF       // Watchdog Timer disabled

#pragma config LVP = OFF        // Low-voltage Programming disabled

#pragma config PBADEN = OFF

unsigned char g_ui8TimerOnflag=0,g_ui8Num=0;

unsigned char g_ui8tmrcnt=0;

void Timer0Init()

{

T0CONbits.T0PS0 = 1; 		// Prescaler of 2

T0CONbits.T0PS1 = 1; 		// Prescaler of 2

T0CONbits.T0PS2 = 1; 		// Prescaler of 2

T0CONbits.PSA = 0;      	// Timer Prescaler assigned

T0CONbits.T0SE = 0;      	// Increment on low to high transition

T0CONbits.T0CS = 0;     	// Internal clock

T0CONbits.T08BIT = 0;   	// 16 BIT MODE

T0CONbits.TMR0ON = 1;   	// Turns timer on

TMR0H =0xB3;

TMR0L = 0xB4;

// Interrupt Setup

RCONbits.IPEN = 1;			// Interrupt priority enabled

INTCONbits.GIE = 1;			// Enables Global Interrupts

INTCONbits.PEIE = 1;		// Enables Peripheral Interrupts

INTCONbits.TMR0IE = 1;		// Enables overflow Interrupt

INTCONbits.TMR0IF = 0; 		// Sets overflow bit to zero

INTCON2bits.TMR0IP = 1;		// Timer 0 Interrupt is high priority

}

void TMRISR (void) ;

#pragma code InterruptVectorHigh=0x08

void InterruptVectorHigh (void)

{

_asm

goto TMRISR

_endasm

}

#pragma code

#pragma interrupt TMRISR

void TMRISR (void) 				//	Timer ISR

{

if(INTCONbits.TMR0IF == 1)

{

INTCONbits.TMR0IF=0;  	//Clear Interrupt Flag to prevent reinitialisation of ISR

TMR0H =0xB3;

TMR0L = 0xB4;

if(g_ui8TimerOnflag)

{

PORTD = 0x00;

g_ui8TimerOnflag=0;

}

else

{

PORTD = 0xff;

g_ui8TimerOnflag=1;

}

}

}

void main (void)

{

Timer0Init();

g_ui8TimerOnflag=0;

TRISD = 0x00;		//Port configured as Output

PORTD = 0x00;		//PORTD = 0x00

while(1)

{}

}